# Dementia Guardian: An IoT-Based Monitoring and Assistance Device for Dementia Patients

## Barath Kumar J    - CB.EN.U4CSE21608

## Shri Praveen S M - CB.EN.U4CSE21658

## Sudharshan B       - CB.EN.U4CSE21661

# Objective

The objective of the Dementia Guardian project is to develop an IoT device that ensures the safety and well-being of dementia patients. The device provides continuous monitoring, real-time location updates, and assistance in case of disorientation. It aims to mitigate the risks associated with wandering and getting lost, common issues faced by dementia patients.

# Hardware Selection and Design

## Sensors, Actuators, Controller, and Communications Units

- ESP32 Microcontroller: Chosen for its integrated Wi-Fi and  Blue- tooth capabilities, low power consumption, and versatility in IoT ap- plications.

- GPS Sensor: Utilized to obtain real-time location data of the patient. This sensor activates when the patient presses the button on the device, providing accurate location information to the caregiver.

- Button: A simple input device for the patient to alert the caregiver and request assistance. Pressing the button sends the patient’s current location to the caregiver.

- Display: A screen to show messages to the patient, such as ”Press the button if you forget” and navigation routes to guide them home.

| Parameters to be Monitored | 1)GPS location (home and current) 2)Button State |
| --- | --- |
| Sensors Used | 1)Neo GPS Module 2)Push Button 3)TFT LCD Display |
| Cloud Platform | Render Cloud Service |
| Web socket Communication | - HTTP REST - FastAPI |
| Database | PostgreSQL |
| Backend /Interface | Python |

# Software Design

## Flowchart / Data Flow Diagram (DFD) / Pseudo Code

### Flowchart

- Start

### Initialize ESP32, GPS, Button, and Display

- Patient presses button

### a. Capture location using GPS

  - b. Send location to caregiver via web application

### c. Display message to patient

- Monitor perimeter

### a. If patient crosses perimeter, alert caregiver

- Record interactions (button presses, perimeter breaches)

### End Pseudo Code

I n i t i a l i z e   ESP32 ,  GPS,   Button ,	Display

while True :

i f   Button  i s	pre s s e d :

l o c a t i o n =  GPS. g e t  l o c a t i o n ()

s e n d l o c a t i o n   t o   c a r e g i v e r ( l o c a t i o n )

Display . show message ( ” Press - the - button - i f - you - f o r g e t ” )

i f	p a t i e n t	c r o s s e s	perimeter :

s e n d a l e r t   t o   c a r e g i v e r ( l o c a t i o n )

Architecture diagram:

r e c o r d i n t e r a c t i o n ( ” perimeter - breach ” ) r e c o r d i n t e r a c t i o n ( ” button - p re s s ” )

DFD Diagram:

# IoT System Design (Level 5 / 6)

## Level 5/6 Design

The IoT system design for the Dementia Guardian device includes:

- End Device (ESP32, GPS, Button, Display): Captures and pro- cesses patient data, sends alerts, and displays messages.

- Edge Computing: Processes data locally on the ESP32, reducing latency and bandwidth usage.

- Cloud Integration: Uses cloud services for data storage, processing, and analysis. The web application for caregivers is hosted on the cloud, providing real-time access to patient data.

# Integration of End, Edge, and Cloud

- End Device: Collects data (location, button presses) and sends it to the Edge.

- Edge Computing: Processes data locally on the ESP32 for immediate actions (alerts, display messages).

- Cloud: Stores data for long-term analysis and provides a web appli- cation for caregivers to monitor patients in real-time.

# Data Accumulation and Processing

- Data Accumulation: All interactions (button presses, perimeter breaches) are recorded and sent to the cloud.

- Processing: Data is processed to generate insights such as frequency of disorientation (button presses) and perimeter breaches.

# IoT Analytics

- Alert Frequency over time : Tracks alerts over the whole period of time

- Alert Source distribution : pie chart of distribution between button alerts and perimeter breach.

- Time between alerts: Analyzes the alert frequency over an internal

- Daily alert patterns : Alert pattern over a daily basis

- Hourly alert trends : alerts in an hourly trends.

# Sample Test Cases and Reports

## Test Case 1: Button Press Functionality

- Objective: Verify that pressing the button sends the correct location to the caregiver.

### Steps:

  - Patient presses the button.

  - Check if the caregiver receives the location update.

  - Confirm that the message is displayed on the device.

- Expected Result: Caregiver receives accurate location, and message is displayed.

## Test Case 2: Perimeter Breach Alert

- Objective: Ensure that crossing the perimeter triggers an alert to the caregiver.

### Steps:

- Patient crosses the predefined safe perimeter.

- Check if the caregiver receives an alert with the location.

- Expected Result: Caregiver receives alert with accurate location.

## Test Case 3: Data Recording

- Objective: Validate that all interactions are recorded accurately.

### Steps:

- Press the button and cross the perimeter multiple times.

- Check the recorded data in the cloud.

- Expected Result: All interactions are logged correctly.

Test Case 4: Medicine Alerts

- Objective: Caregiver set the period of interval between timely alerts.

### Steps:

- Set the interval and medicine names.

- Check for alerts in hardware display when time is up.

- Expected Result: Repeated periodic alerts or medicine

Test Case 5: Patient Current Location

- Objective: To check patients current location by the caregiver at any point of time.

### Steps:

- Patient device to be on so it can send location periodically.

- Caregiver to check in his webapp for the current location

- Expected Result: Accurate realtime current location of patient.

# Outcomes

- Real-time Monitoring: Continuous tracking of patient location en- sures their safety.

- Immediate Assistance: Patients can easily request help by pressing a button.

- Caregiver Alerts: Instant notifications for perimeter breaches and disorientation incidents.

- Data Insights: Analysis of interaction data aids in understanding patient behavior and improving care.

- Medication Reminders: Adding functionality to remind patients of their medication schedules.

- Monitor patient Current Location: Caregiver can constantly check the patient’s current location

# Conclusion and Future Scope

## Conclusion

The Dementia Guardian project successfully developed an IoT device that enhances the safety and well-being of dementia patients through real-time monitoring, location updates, and caregiver alerts. The system’s ability to record and analyze patient interactions provides valuable insights for improv- ing patient care.

## Future Scope

- Navigation Assistance: Implementing navigation routes on the dis- play to guide patients home.

- Task Reminders: Integrating reminders for daily tasks and appoint- ments to further support patient independence.